|  | School of Computing Faculty of Engineering UNIVERSITI TEKNOLOGI MALAYSIA |
| --- | --- |

| SUBJECT NAME: | COMPUTER ORGANIZATION AND ARCHITECTURE |
| --- | --- |
| SUBJECT CODE: | SCSR/SECR 2033 |
| SEMESTER: | 2019/20-2 |
| LAB TITLE: | Programming 3a: Flags, OFFSET, Arrays, JMP, LOOP |
| INSTRUCTION: | Answer all questions. |
| STUDENT INFO : | Name: HAM JING YI Matric No: A19EC0048 Section: 02 Email: hamjingyi@graduate.utm.my |
| SUBMISSION DATE: 30/4/2020 |  |

| COMMENTS: |  |
| --- | --- |

Part A – Programming review

Flags Affected by Arithmetic

- The ALU has a number of status flags that reflect the outcome of arithmetic (and bitwise) operations

o based on the contents of the destination operand

- Essential flags:

o Zero flag – set when destination equals zero o Sign flag – set when destination is negative

o Carry flag – set when unsigned value is out of range

  - Overflow flag – set when signed value is out of range

- The MOV instruction never affects the flags.

Zero Flag (ZF)

- The Zero flag is set when the result of an operation produces zero in the

| destination operand. |  |  |  |
| --- | --- | --- | --- |
| mov ax,2 |  |  |  |
| sub ax,1 |  | ; AX = 1, | ZF = 0 |
| mov cx,1 |  |  |  |
| sub cx,1 |  | ; CX = 0, | ZF = 1 |
| mov ax,0FFFFh |  |  |  |
| inc ax | ; AX = 0, ZF = |  | 1 |
| inc ax | ; AX = 1, ZF = |  | 0 |

Sign Flag (SF)

- The Sign flag is set when the destination operand is negative. The flag is

clear when the destination is positive.

mov cx,0

sub cx,1	; CX = -1, SF = 1

add cx,2	; CX = 1, SF = 0

	The sign flag is a copy of the destination's highest bit:

mov al,0

sub al,1	; AL = 11111111b, SF = 1

add al,2	; AL = 00000001b, SF = 0

Overflow and Carry Flags: A Hardware Viewpoint

- How the ADD instruction modifies OF and CF:

  - OF = (carry out of the MSB) XOR (carry into the MSB)

  - CF = (carry out of the MSB)

- How the SUB instruction modifies OF and CF:

  - NEG the source and ADD it to the destination

hkm/S11415

o OF = (carry out of the MSB) XOR (carry into the MSB)

o CF = INVERT (carry out of the MSB)

MSB = Most Significant Bit (high-order bit)

XOR = eXclusive-OR operation

NEG = Negate (same as SUB 0,operand)

Carry Flag (CF)

- The Carry flag is set when the result of an operation generates an unsigned value that is out of range (too big or too small for the

destination operand). mov al,0FFh

add al,1	; CF = 1, AL = 00

; Try to go below zero:

mov al,0

sub al,1	; CF = 1, AL = FF

Overflow Flag (OF)

- The Overflow flag is set when the signed result of an operation is invalid or out of range.

; Example 1

mov al,+127

add al,1	; OF = 1,	AL = 80h

; Example 2

mov al,7Fh

add al,1	; OF = 1,	AL = 80h

- The two examples are identical at the binary level because 7Fh equals +127. To determine the value of the destination operand, it is often easier to calculate in hexadecimal.

**NOTE: In VisualStudio Register Window during Step Over, you will find that the flag registers are presented with a different name.

Overflow Flag (OF) = OV

Zero Flag (ZF) = ZR

Sign Flag (SF) = PL

Parity Flag (PF) = PE

Carry Flag (CF) = CY

Auxiliary Flag (AF) = AC

hkm/S11415

OFFSET Operator

- OFFSET returns the distance in bytes, of a label from the beginning of its enclosing segment

o Protected mode: 32 bits o Real mode: 16 bits

- OFFSET gives you the address where the variable (or an array) starts.

offset

Data segment:

myByte

- Example:  Let's assume that the data segment begins at 00404000h:

|  |  |  | .data |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | bVal BYTE 10h |  |  |  |  |  |  |  |  |
|  |  |  | wVal WORD 1000h |  |  |  |  |  |  |  |  |
|  |  |  | dVal DWORD 10001000h |  |  |  |  |  |  |  |  |
|  |  |  | dVal2 DWORD ? |  |  |  |  |  |  |  |  |
|  |  |  | .code |  |  |  |  |  |  |  |  |
|  |  | mov esi,OFFSET bVal |  |  |  |  |  | ; ESI = 00404000 |  |  |  |
|  |  | mov ah, bVal |  |  |  |  |  | ; AH = 10h |  |  |  |
|  |  | mov esi,OFFSET wVal |  |  |  |  |  | ; ESI = 00404001 |  |  |  |
|  |  | mov ax, wVal |  |  |  |  |  | ; AX = 1000h |  |  |  |
|  |  | mov esi,OFFSET dVal |  |  |  |  |  | ; ESI = 00404003 |  |  |  |
|  |  | mov eax, dVal |  |  |  |  |  | ; EAX = 10001000h |  |  |  |
|  |  | mov esi,OFFSET dVal2 |  |  |  |  |  | ; ESI = 00404007 |  |  |  |
|  | 00404000 |  |  | 00404001 |  | 00404003 |  | 00404007 |  |  |  |

bVal	wVal	dVal	dVal2

+1	+2	+4

**TIP: Note the different outcomes in the MOV instructions with and without the use of OFFSET.

hkm/S11415

Arrays

- Arrays are probably the most commonly used composite data type.

- Analogy:

  - An array is like a drawer that holds many items of the same type. Like a sock drawer that have 10 different pairs of socks, and you can reach these socks from that drawer.

- Defining an array:

  - Must have array name, size of each item in array, initialize (or not ) the values of these items o Example:

| Array1 |  | byte | 10h, 20h, 35h |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ; 3 | items in array, each |  |  |  |  | 1 | byte in size |  |  |  |  |  |
| Array2 |  | word 25h, 1A20h, |  |  |  | 66h, 891h |  |  |  |  |  |  |
| ; 4 | items in array, each |  |  |  |  | 2 | bytes in size |  |  |  |  |  |
| Array3 word 4 dup (0) |  |  |  |  |  |  |  |  |  |  |  |  |
| ; 4 | items in array, each |  |  |  |  | 2 | bytes , and initialized to 0 |  |  |  |  |  |
|  | Array1 |  | 10h | 20h |  | 35h |  |  |  |  |  |  |
|  | Array2 |  | 0025h |  |  | 1A20h |  |  |  | 0066h | 0891h |  |

- Handling an array:

- Use register as a pointer, the method is called indirect addressing.

  - Traversing an array (i.e. moving through an array), the pointer must be incremented following the array type (byte [+1] or word [+2] or dword[+4]).

- Example:

.data

Array1 byte 10h, 20h, 35h	;Array1 starts at address 404000

Array2 word 25h, 1A20h, 66h, 891h

.code main PROC

; Calling array method 1

| mov esi,OFFSET | Array1 | ; ESI = 00404010 |
| --- | --- | --- |
| mov al, [esi] | ; AL = 10h |  |
| add esi,1 | ; ESI = 00404001 |  |
| mov bl, [esi] | ; BL = 20h |  |
| mov esi,OFFSET | Array2 | ; ESI = 00404003 |
| mov ax, [esi] | ; AX = 0025h |  |
| add esi,2 | ; ESI = 00404005 |  |
| mov bx, [esi] | ; BX = 1A20h |  |

hkm/S11415

| ; Calling array method |  | 2 |
| --- | --- | --- |
| mov al, Array1 | ; | AL = 10h |
| mov bl, Array1+1 | ; | BL = 20h |
| mov ax, Array2 | ; | AX = 0025h |
| mov bx, Array2+2 | ; | BX = 1A20h |

**TIP: You can also use TYPE <array name> to match the array type.

Example: mov bl, Array1+type Array1; BL = 20h

	Example: Sum an array

.data

Array1 byte 10h, 20h, 35h

Array2 word 25h, 1A20h, 66h, 891h

.code main PROC

| ;Sum an | array |  |
| --- | --- | --- |
| mov al, | Array1 | ; AL = 10h |
| add al, | Array1+1 | ; AL = 30h |
| add al, | Array1+2 | ; AL = 65h |
| mov bx, | Array2 | ; BX = 0025h |
| add bx, | Array2+2 | ; BX = 1A45h |
| add bx, | Array2+4 | ; BX = 1AABh |
| add bx, | Array2+6 | ; BX = 233Ch |

Jump

- To jump here means to relocate the instruction pointer to a different address, one that is not sequential (i.e. not the next one).

- Jumps can be

- Conditional: jump when a condition(s) is met; if not met don’t jump. Conditions can be flags, arithmetic results, etc.

  - Example: JNZ (Jump Not Zero), JE (Jump Equal), JB (Jump Below)

  - Unconditional: no conditions, you MUST jump

    - Example: JMP

- Let’s explore unconditional jumps with JMP

- The JMP command causes unconditional transfer to a destination (label) that is usually within the same procedure.

hkm/S11415

o The command format is JMP destination

o The destination is a label

o When this command is executed, the instruction pointer (EIP)

will now point to the address where the label (or the destination) is.

0001	MOV AX,10

0002

0003	HERE:

0004	INC AX

0005	JMP HERE

0006	ADD AX,2

	When the command INC AX is executed, EIP points to 0005.

	When the command JMP HERE is executed, EIP will now point to 0003 rather than 0006

	The problem here: this is an endless loop

 Let’s explore a conditional jump example with JNZ. o The format is the same as unconditional jump.

o  JNZ  jump to a label if the Zero flag is clear [ZF = 0]

- In the example below, once the Zero Flag is set [ZF = 1], the condition for JNZ is not met; so it will not be done. EIP will point to 0006.

| 0001 | MOV AX,10 |
| --- | --- |
| 0002 |  |
| 0003 | HERE: |
| 0004 | DEC AX |
| 0005 | JNZ HERE |
| 0006 | ADD AX,2 |

- Please do explore the different conditional jumps that are available to you.

o Conditional jumps are usually accompanied with a compare (CMP) command.

o More of these in upcoming labs.

LOOP

- As the name implies, the LOOP instruction will repeatedly execute a block of statements.

- The number of time the looping will occur is held in a counter. In a 32-bit mode, the counter is the register ECX.

- The loop instruction decrements register ECX and compares it with 0 leaving the flags unchanged.

o If new ECX ≠ 0, jumps to the label.

o Else, the program execution continues with the next instruction.

- The command format is LOOP destination

o The destination is a label

hkm/S11415

- Example: try and trace the program below.

.code main PROC

mov eax,10h

mov ecx, 4 ; ecx is the counter

L1:

add eax,2	; eax = eax + 2

loop L1	; ecx = ecx -1; go to L1 if ECX ≠ 0

|  | exit |  |  |  |
| --- | --- | --- | --- | --- |
|  | main ENDP |  |  |  |
|  | END main |  |  |  |
|  | ECX |  | EAX |  |
|  | 4 (initial value) |  | 10h (initial value) |  |
|  |  | add eax,2 | 12h |  |
|  | 3 | loop L1  go to L1 |  |  |
|  |  | add eax,2 | 14 |  |
|  | 2 | loop L1  go to L1 |  |  |
|  |  | add eax,2 | 16 |  |
|  | 1 | loop L1  go to L1 |  |  |
|  |  | add eax,2 | 18 |  |

  - loop L1  Stop

- You can do a nested loop instruction (if need be).

- If you need to code a loop within a loop, you must save the outer loop counter's ECX value.

hkm/S11415

Part B – Let’s do a little programming

- Given the assembly language program below, run it and list the flags’ status after each instruction.

| PROGRAM |  | OF(OV) |  |  | SF(PL) |  |  | ZF(ZR) |  |  | AF(AC) |  |  | PF(PE) |  |  | CF(CY) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 |  |  | 0 |  |  | 1 |  |  | 0 |  |  | 1 |  |  | 0 |  |  |
| mov ax,10h |  | 0 |  |  | 0 |  |  | 1 |  |  | 0 |  |  | 1 |  |  | 0 |  |
| add ax,2h |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 1 |  |  | 0 |  |
| sub ax,15h |  | 0 |  |  | 1 |  |  | 0 |  |  | 1 |  |  | 0 |  |  | 1 |  |
| add ax,112 |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 1 |  |
| neg ax |  | 0 |  |  | 1 |  |  | 0 |  |  | 1 |  |  | 1 |  |  | 1 |  |
| mov bh,66h |  | 0 |  |  | 1 |  |  | 0 |  |  | 1 |  |  | 1 |  |  | 1 |  |
| inc bx |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 1 |  |
| mul dh |  | 1 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 1 |  |  | 1 |  |
| sub al,3 |  | 0 |  |  | 0 |  |  | 0 |  |  | 1 |  |  | 1 |  |  | 0 |  |

- What will be the values of the Overflow flag in the program given below?

mov al,80h

add al,92h ; AL = 12h , OF = 1

mov al,-2

add al,+127 ; AL = 7Dh , OF = 0

- Define the following arrays:

  - A byte type array named PKP with 3 items 11, 22h and 4Ah.

PKP byte 11, 22h, 4Ah

  - A word type array named ZOOM with 5 items 45, 45h, 444h, 4A4Bh and 44Ah.

ZOOM word 45, 45h, 444h, 4A4Bh, 44Ah

c.    A double-word type array named PADLET with 5 items initialized to 0

PADLET dword 5 dup (0)

hkm-mma20192020-2

- Referring to the array definitions in Question 3, state the following values in the register.

| a. | MOV AL, PKP | ; AL =0Bh |
| --- | --- | --- |
| b. | MOV AL, PKP+3 | ; AL =2Dh |
| c. | MOV AX, ZOOM | ; AX =2Dh |
| d. | MOV AX, ZOOM+3 | ; AX =4400h |
| e. | MOV AX, ZOOM+4 | ; AX =0444h |
| f. | MOV EAX, PADLET | ; EAX =00000000h |
| g. | MOV AH, PKP+8 | ; AH =04h |
|  | MOV EAX,0 | ; EAX =0000000h |
| h. | MOV AX, ZOOM+2 | ; AX =0045h |
|  | MOV PADLET, EAX | ; PADLET =00000045h |

- Referring to the array definitions in Question 3, write the appropriate instruction(s) to achieve the required results.

| a. | mov bx, ZOOM+2 | ; BL = 45h |
| --- | --- | --- |
| b. | mov bx, 0 mov bl, PKP+2 | ; BX = 004Ah |
| c. | mov eax, 0 mov ax, ZOOM+6 mov PADLET+4, eax | ; PADLET+4 = 00004A4Bh |

- (video) Referring to the array definitions in Question 3, write a program to sum array PKP.

- (video) Referring to the array definitions in Question 3, write a program to sum array ZOOM.

hkm/S11415

Program of Q6

TITLE MASM Template (main.asm) ; Description: Q6 Part B ; ; Revision date: INCLUDE Irvine32.inc .data PKP byte 11, 22h, 4Ah total byte ? .code main PROC ;Sum an array mov al, PKP add al, PKP+1 add al, PKP+2 mov total, al exit main ENDP END main

Program of Q7

TITLE MASM Template (main.asm) ; Description: Q7 Part B ; ; Revision date: INCLUDE Irvine32.inc .data ZOOM word 45, 45h, 444h, 4A4Bh, 44Ah total word ? .code main PROC mov bx, ZOOM add bx, ZOOM+2 add bx, ZOOM+4 add bx, ZOOM+6 add bx, ZOOM+8 mov total, bx exit main ENDP END main

- (video) Study the assembly instructions given below and fill in the blanks (in hexadecimal).

INCLUDE Irvine32.inc

.data

intArray WORD 100h,200h,300h,400h

TOTAL WORD 0

.code main PROC

mov edi,OFFSET intArray	; EDI = 00404000

mov ecx,LENGTHOF intArray	; ECX = 00000004

mov ax,0 L1:

add ax,[edi]

add edi,TYPE intArray ; EDI = EDI + 2

loop L1

mov TOTAL, ax	; TOTAL = 0A00h

exit main ENDP

END main

9.	Study the assembly instructions given below and answer the following questions.

mov ax,20

mov ecx,4

L1:

inc ax neg ax loop L1

- How many times will the loop be executed? __4___

- What is the final result of AX in hexadecimal? ___0014h____

- Fill in the table with the value of AX after each instruction in each loop.

| Loop# | INC AX | NEG AX |
| --- | --- | --- |
|  | Initially AX = 20 |  |
| 1 | 21 or 0015h | 65515 or FFEBh |
| 2 | 65516 or FFECh | 20 or 0014h |
| 3 | 21 or 0015h | 65515 or FFEBh |
| 4 | 65516 0r FFECh | 20 or 0014h |

hkm/S11415

- Study the assembly language code given below. What is the final value of the variable TOTAL? ____00000070h____

INCLUDE Irvine32.inc

.data

total dword 0 counter dword 7

.code main PROC

mov Eax,0

mov Ecx,counter L1:

add Eax,10h loop L1

mov total, eax

exit main ENDP

END main

- (video) Using the LOOP instruction, write a program to achieve the following equation. What is the final result of TOTAL in hexadecimal and decimal? ___00000700h or 1792d_____

TOTAL = 100h * 7h

TITLE MASM Template (main.asm) ; Description: Q10 Part B ; ; Revision date: INCLUDE Irvine32.inc .data total dword 0 counter dword 7 .code main PROC mov eax,0 mov ecx,counter L1: add eax,100h loop L1 mov total, eax exit main ENDP END main