Department of Computer

Science

Faculty of Comput

ing

UNIVERSITI TEKNOLOGI MALAYSIA

MARKS:

SUBJECT : SCSR1013 DIGITAL LOGIC SESSION/SEM : ………………………………………………………………….

LAB 2 : OMBINATIONAL LOGIC CIRCUIT DESIGN SIMULATION

NAME 1 ：Singthai Srisoi DATE : 9.1.2021

REMARKS 	:

D.  Lab Activities

# Part 1

Simulating logic circuit, construct truth table and timing diagram with Deeds.

Given Boolean expression as follow:

Y=AB+BC+AC

- Convert the non-standard Boolean expression into standard form.

- Based on standard form expression, complete the following truth table.

|  | INPUT |  | OUTPUT |
| --- | --- | --- | --- |
| A | B | C | Y |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |

- Using Deeds Simulator, draw the following circuits:

- Circuit (i) for non-standard form (based on the given expression).

Circuit (i)

- Circuit (ii) for standard form (from your answer in question (1)).

Circuit (ii)

- Simulate these two circuits in step (3) and complete their truth table.

Compare the simulation result for these two truth tables. What is your conclusion?

Circuit (i)

|  | INPUT |  | OUTPUT |
| --- | --- | --- | --- |
| A | B | C | Y |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |

Circuit (ii)

|  | INPUT |  | OUTPUT |
| --- | --- | --- | --- |
| A | B | C | Y |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |

Conclusion:

The truth table of circuit 1 and circuit 2 are the same. Hence circuit 1 is the simpler version of circuit 2.

- Simulate output of circuit (ii) with Timing Diagram. Illustrate some examples of different inputs and output.

# Part 2

Combinational circuit design process and simulate with Deeds Simulator.

## Design Process

- Determine Parameter Input / Output and their relations.

- Construct Truth Table. iii) 	Using K-Map, get the SOP optimized form of all Boolean equation outputs.

- Draw the circuit and use duality symbol; convert AND-OR circuit to NAND gates ONLY.

- Simulate the design using Deeds Simulator. Check the results according to Truth Table and Timing Diagram Operation.

## Problem Situation

A new digital fault diagnoses circuit is requested to be designed for analyzing four bit 2’s complement input binary number from sensors A, B, C, and D. Sensor A represents input MSB and sensor D represents input LSB. As shown in the following Figure 5, bit pattern analysis from input sensors A, B, C, and D will trigger four different output errors (active HIGH) of type E1, E2, E3, and E4.

## Figure 5

The following rules are used to activate the error’s signal type:

| RULE 1: | E1 is activated if the input number is positive ODD and the majority of the bits is ‘0’. |
| --- | --- |
| RULE 2: | E2 is activated if the input number is positive EVEN and the majority of the bits is ‘0’. |
| RULE 3: | E3 is activated if the input number is negative ODD and the majority of the bits is ‘1’. |
| RULE 4: | E4 is activated if the input number is negative EVEN and the majority of the bits is ‘1’. |
| RULE 5: | The output of error signal is invalid if the input has equal bit ‘0’ |

and bit ‘1’.

(NOTE:         Positive ODD is positive numbers that are odd and negative

EVEN is   negative numbers that are even).

## Experimental Steps

1.  Complete Truth Table 1 for Digital Fault Diagnose Circuit. Use variables A, B, C and D as inputs; E1, E2, E3 and E4 as outputs.

## Truth Table 1

|  | INPUTS |  |  |  | OUTPUTS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A | B | C | D | E1 | E2 | E3 | E4 |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | x | x | x | x |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |
| 0 | 1 | 0 | 1 | x | x | x | x |
| 0 | 1 | 1 | 0 | x | x | x | x |
| 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 1 | x | x | x | x |
| 1 | 0 | 1 | 0 | x | x | x | x |
| 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 | x | x | x | x |
| 1 | 1 | 0 | 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 |

- Using K-MAP, get minimized SOP Boolean expressions for E1, E2, E3 and E4 circuits.

- From the Boolean expression in the step (2), draw your final E1, E2, E3 and E4 circuits using 2 input basic gates (AND, OR, NOT). Use Deeds Simulator.

- Simulate the Deeds circuit in step (3):

a) Update Truth Table 2 based on the simulation result.

## Truth Table 2

|  | INPUTS |  |  |  | OUTPUTS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A | B | C | D | E1 | E2 | E3 | E4 |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |
| 0 | 1 | 0 | 1 | 1 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 | 0 | 0 | 1 |
| 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |
| 1 | 1 | 0 | 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 |

Compare the output results in Truth Table 2 with Truth Table 1. What is your conclusion?

Truth table 2 did not include the don’t care condition

b) Timing Diagram

Explain some analysis values based on your timing diagram:

E1 is the inverse of E2 for the first eight output

E3 is the inverse of E4 for the last eight output

- Using dual symbol concept, convert your circuit in step (3) to NAND gates only. Use Deeds Simulator.

- Simulate the Deeds circuit in step (5):

a) Update Truth Table 3 based on the simulation result.

## Truth Table 3

|  | INPUTS |  |  |  | OUTPUTS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A | B | C | D | E1 | E2 | E3 | E4 |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |
| 0 | 1 | 0 | 1 | 1 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 | 0 | 0 | 1 |
| 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |
| 1 | 1 | 0 | 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 |

Compare the output results in Truth Table 3 with Truth Table 2. What is your conclusion?

Table 3 and table 2 are the same. Hence the circuit is the same.

b) Timing Diagram

Explain some analysis values based on your timing diagram:

E1 is the inverse of E2

E3 is the inverse of E4

|  | Partially Completed |  |
| --- | --- | --- |

Fully

Completed  Checked by: __________