LAB 3 : MUHAMMAD AMIRUDDIN BIN ZULKIFLI | A20EC0208

AMIRUL IMAN BIN AHMAD KEFLEE | A20EC0183

E. Preliminary Works

1.

| Desired Result | PRE’ | CLR’ | J | K | CLK | Q |
| --- | --- | --- | --- | --- | --- | --- |
| Set Initial value Q=1 | 1 | 1 | X | X | -- | 1 |
| Output Q stays the same | 1 | 1 | 0 | 0 |  | 1 |
| Output Q become 0, NC in asynchronous input | 1 | 0 | X | X |  | 0 |
| Output Q is not the previous Q | 1 | 1 | 1 | 0 |  | 1 |
| RESET Q | 1 | 1 | 0 | 1 |  | 0 |
| SET Q | 1 | 1 | 1 | 0 |  | 1 |

2. a) Which state that JK FF has but not on SR FF?

Toggle

b) Identify whether the JK FF in 7476, is a positive-edge triggered or negative-edge triggered FF.

Negative-edge triggered Flip Flop