School of Computing

Faculty of Engineering	of	Comput

UNIVERSITI	TEKNOLOGI	MALAYSIA

| SUBJECT | : SECR1013 DIGITAL LOGIC |
| --- | --- |
| SESSION/SEM | : …2021/1………………………………………………………………. |
| LAB 3 | : SYNCHRONOUS DIGITAL COUNTER |
| NAME | : ………Tan Ming Hui…………………………………………………………. |
| DATE | : ……………2021/01/18……………………………………………………. |

Lab #3

Identifying the Properties of a Synchronous Counter

A. Aims

- Expose the student with experience on constructing synchronous counter circuit using Flip-Flop IC, Basic Gate ICs, Breadboard and ETS-5000 Digital Kit.

- Promote critical thinking among students by analysing the given circuit and identifying the behaviour of the digital circuit.

B. Objectives

The objectives of this lab activity are to:

- Implement a synchronous counter circuit into physical circuit using Breadboard, Flip-Flops, Basic Gates and Switches.

- Completing the next-state table of the counter circuit.

- Sketch the state diagram of the counter circuit.

- Identify the properties of the counter.

C. Materials And Equipment

Materials and equipment required for this lab are as follows:

| Item Name | Number of Item |
| --- | --- |
| 1. Breadboard | 1 |
| 2. 7408 Quad 2-Input AND | 1 |
| 3. 7404 Hex Inverter | 1 |
| 4. 7432 Quad 2-input OR | 1 |
| 5. 7476 Dual J-K Flip Flop | 1 |
| 6. ETS-5000 Digital Kit | 1 |

D. Preliminary Works

1) Determine the logic level for each input combinations in Table 1 so that the desired result can be realized.

Table 1

| Desired Result | PRE | CLR | J | K | CLK | Q |
| --- | --- | --- | --- | --- | --- | --- |
| Set initial value Q = 1 | 0 | 1 | X | X | -- | 1 |
| Output Q stays the same | 1 | 1 | 0 | 0 | ⇓ | 1 |
| Output Q become 0, no change in asynchronous input | 1 | 1 | 0 | 1 | ⇓ | 0 |
| Output Q is not the previous Q | 1 | 1 | 1 | 1 | ⇓ | 1 |
| RESET Q | 1 | 1 | 0 | 1 | ⇓ | 0 |
| SET Q | 1 | 1 | 1 | 0 | ⇓ | 1 |

2) Answer all questions.

- Which state that JK flip-flop has, but not on SR flip-flop.

When J and K both are in the state of High (1), JK flip-flop have the output of toggle. However, when S and R both are in the state of High (1), SR flip-flop is invalid.

- Identify whether the JK flip flop in 7476, is a positive-edge triggered or negative-edge triggered flip flop.

- negative-edge triggered flip flop because the arrow shown is to downward

E. Lab Activities

1) You are given a counter circuit as shown in Figure 4.

Figure 4: A Synchronous Counter Circuit

- By using all materials and equipment’s listed in section C, construct the physical circuit of Figure 4. (Make sure all ICs are connected to Vcc and GND).

- Investigate the behaviour of the counter by observing the next state of the counter for all combination of Present State and X values. Complete the NextState table of the counter in Table 2. Ensure the Switch 0 is in HIGH state.

(0=LOW, 1=HIGH)

Table 2

| Switch 7 | Present State |  | Next State |  |
| --- | --- | --- | --- | --- |
| X | Q1 LED 1 | Q0 LED 0 | Q1 LED 1 | Q0 LED 0 |
| 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 |

- By referring to the Next-State in Table 2, sketch the state diagram of the counter.

1

00

01

10

11

0

0

0

0

1

1

1

- By referring to the Next-State in Table 2 and the state diagram in (4), answer all questions.

  - What is the main indicator to decide that the counter is a synchronous counter?

The clock are from the same source (Pulser Switch A’).

  - How many states are available for the counter and what are they?

4 stage which are 00,01,10,11

  - What is the function of Switch 7 (X) in the circuit?

Switch 7(X) is to control the JK flip-flop, to make the counter wheter is count-up, or count-down, or count-up and down.

  - What is the function of Switch 0 and Switch 1 in the circuit?

Switch 0 is CLR that have the function to reset the output while switch 1 is Preset that have the functon of set the output.

  - Is the counter a saturated counter or recycle counter?

Saturated counter because 00, and 11  are continue to itself when count down and count-up respectively.

- Referring to state diagram in 4, draw and built a synchronous counter using D flip-flop.

  - Built the next state and transition table using the header in Table 3

Table 3

| Input X | Present State |  | Next State |  | D FF Transition |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Q1 | Q0 | Q1+ | Q0+ | D1 | D0 |
| 0 | 0 | 0 | 0 | 1 | 0 | 1 |
| 0 | 0 | 1 | 1 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 | 1 | 1 | 1 |
| 0 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 | 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 | 1 | 0 |

  - Get the optimized Boolean expression.

D0						D1

| Q1Q0 X | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 1 | 1 |
| 1 | 0 | 0 | 0 | 1 |

| Q1Q0 x | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 1 |

000	011	010				001	011	010

010	010	110				011	010	110

0  0	01	  10				0  1	01	  10

X’ Q0’	x’Q1	Q1Q0’				x’Q0	x’Q1	   Q1Q0

D0 = X’ Q0’ + X’Q1 + Q1Q0’		D1 = X’Q0 + X’Q1 + Q1Q0

  - Draw the complete final circuit design in Deeds.

*In the file of submition name (TanMingHui_04_Q6.pbs)

- Repeat steps in Q(6) using T flip-flop.

- table

| Input X | Present State |  | Next State |  | T FF Transition |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Q1 | Q0 | Q1+ | Q0+ | T1 | T0 |
| 0 | 0 | 0 | 0 | 1 | 0 | 1 |
| 0 | 0 | 1 | 1 | 0 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 | 1 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 | 1 | 1 |
| 1 | 1 | 1 | 1 | 0 | 0 | 1 |

- b) boolean expression

- T0							T1

| Q1Q0 X | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 1 | 1 | 1 |

| Q1Q0 X | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 |

- 011= X+Q1’+Q0’						001 = X’Q1’Q0

- 100 = X’ + Q1 + Q0						110 = XQ1Q0’

- T0=(X+Q1’+Q0’) (X’ + Q1 + Q0)				T1= (X’Q1’Q0) + ( XQ1Q0’ )

- c) circuit

- * In the file of submition name (TanMingHui_04_Q7.pbs)