Department of Computer Science

Faculty of Computing

UNIVERSITI TEKNOLOGI MALAYSIA

SUBJECT : SCSR1013 DIGITAL LOGIC SESSION/SEM: 2020-21-01

LAB 2 : COMBINATIONAL LOGIC CIRCUIT DESIGN SIMULATION

Group No: 04. NAME 1 : ABU SAKIB BIN LUTFUL HASSAN (A20EC4007). NAME 2 : LIM LI TING (A20EC5008). NAME 3 : MD FARIDUL ISLAM (A20EC4030).

REMARKS 	:

MARKS:

D. Lab Activities

# Part 1

Simulating logic circuit, construct truth table and timing diagram with Deeds.

Given Boolean expression as follow:

Y = AB + BC + AC

- Convert the non-standard Boolean expression into standard form.

Y=AB (C+ C̅) +BC (A+A̅) +AC (B+B̅)

=ABC+ABC̅ +ABC+A̅BC+ABC+AB̅ C

- Based on standard form expression, complete the following truth table.

|  | INPUT |  | OUTPUT |
| --- | --- | --- | --- |
| A | B | C | Y |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |

- Using Deeds Simulator, draw the following circuits:

- Circuit (i) for non-standard form (based on the given expression).

Circuit (i)

- Circuit (ii) for standard form (from your answer in question (1)).

Circuit (ii)

# Part 2

Combinational circuit design process and simulate with Deeds Simulator.

## Design Process

- Determine Parameter Input / Output and their relations.

- Construct Truth Table. iii) Using K-Map, get the SOP optimized form of all Boolean equation outputs.

- Draw the circuit and use duality symbol; convert AND-OR circuit to NAND gates ONLY.

- Simulate the design using Deeds Simulator. Check the results according to Truth Table and Timing Diagram Operation.

## Problem Situation

A new digital fault diagnoses circuit is requested to be designed for analyzing four bit 2’s complement input binary number from sensors A, B, C, and D. Sensor A represents input MSB and sensor D represents input LSB. As shown in the following Figure 5, bit pattern analysis from input sensors A, B, C, and D will trigger four different output errors (active HIGH) of type E1, E2, E3, and E4.

## Figure 5

The following rules are used to activate the error’s signal type:

| RULE 1: | E1 is activated if the input number is positive ODD and the majority of the bits is ‘0’. |
| --- | --- |
| RULE 2: | E2 is activated if the input number is positive EVEN and the majority of the bits is ‘0’. |
| RULE 3: | E3 is activated if the input number is negative ODD and the majority of the bits is ‘1’. |
| RULE 4: | E4 is activated if the input number is negative EVEN and the majority of the bits is ‘1’. |
| RULE 5: | The output of error signal is invalid if the input has equal bit ‘0’ and bit ‘1’. |
| (NOTE: | Positive ODD is positive numbers that are odd and negative EVEN is negative numbers that are even). |

## Experimental Steps

1. Complete Truth Table 1 for Digital Fault Diagnose Circuit. Use variables A, B, C and D as inputs; E1, E2, E3 and E4 as outputs.

## Truth Table 1

|  | INPUTS |  |  |  | OUTPUTS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A | B | C | D | E1 | E2 | E3 | E4 |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | x | x | x | X |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |
| 0 | 1 | 0 | 1 | x | x | x | X |
| 0 | 1 | 1 | 0 | x | x | x | x |
| 0 | 1 | 1 | 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | x | x | x | X |
| 1 | 0 | 1 | 0 | x | x | x | x |
| 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 | x | x | x | x |
| 1 | 1 | 0 | 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 |

2. Using K-MAP, get minimized SOP Boolean expressions for E1, E2, E3 and E4 circuits.

CD

00

01

11

10

AB

00

01

11

10

E2

)

i

(

E1

i

)

(

X

E2

E2

)

(

i

X

X

X

E3

(

i

)

E3

(1)

E4

i

)

(

0

X

E3

(

i

)

X

E1= A̅ B̅ C̅ D

E2= AB̅ C̅ D̅ + A̅ B̅ CD̅

E3= ABD + AB̅ CD

E4= ABCD̅

3.From the Boolean expression in the step (2), draw your final E1, E2, E3 and E4 circuits using 2 input basic gates (AND, OR, NOT). Use Deeds Simulator.

E1:

E2:

E3:

E4:

4.Simulate the Deeds circuit in step (3):

a) Update Truth Table 2 based on the simulation result.

## Truth Table 2

|  | INPUTS |  |  |  | OUTPUTS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A | B | C | D | E1 | E2 | E3 | E4 |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 |

Compare the output results in Truth Table 2 with Truth Table 1. What is your conclusion?

# Same result as the equations were derived from the previous table.

b) Timing Diagram:

E1:

E2:

E3:

E4:

Explain some analysis values based on your timing diagram:

# In E1: only input D value is come out as an output; for E2: if all input is low

Then output will be high. And for E3: if all input is high then output will be high. And    lastly   For E4: if the input for A, B, C is high and D is low then the output will be

High.

5.

Using dual symbol concept, convert

your circuit in step (3) to NAND gates only. Use

Deeds

Simulator.

E1:

E2:

E3:

E4:

6.Simulate the Deeds circuit in step (5):

a) Update Truth Table 3 based on the simulation result.

## Truth Table 3

|  | INPUTS |  |  |  | OUTPUTS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A | B | C | D | E1 | E2 | E3 | E4 |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | x | x | x | X |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |
| 0 | 1 | 0 | 1 | x | x | x | X |
| 0 | 1 | 1 | 0 | x | x | x | x |
| 0 | 1 | 1 | 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | x | x | x | X |
| 1 | 0 | 1 | 0 | x | x | x | x |
| 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 | x | x | x | x |
| 1 | 1 | 0 | 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 |

Compare the output results in Truth Table 3 with Truth Table 2. What is your conclusion?

Same because NAND gates are universal gates. It has been used to replace

The basic gates here.

b) Timing Diagram

E1:

E2:

E3:

E4:

Explain some analysis values based on your timing diagram:

# The timing diagram shows that the input value for A, B, C, D for each circuit

E1, E2,E3 and E4 . And show the output as well.

Fully Partially

Completed Completed Checked by: