| School of Computing Faculty of Engineering UNIVERSITI TEKNOLOGI MALAYSIA |  |  |
| --- | --- | --- |
|  | SUBJECT | : SECR1013 DIGITAL LOGIC |
|  | LAB 3 | : SYNCHRONOUS DIGITAL COUNTER |
|  | NAME | : Shabrina Salsabila Sakroni (A20EC0334) Tanshiba Naorin Prapti (A20EC4051) |

Lab #3

Identifying the Properties of a Synchronous Counter

- Aims

  - Expose the student with experience on constructing synchronous counter circuit using Flip-Flop IC, Basic Gate ICs, Breadboard and ETS-5000 Digital Kit.

  - Promote critical thinking among students by analysing the given circuit and identifying the behaviour of the digital circuit.

# Objectives

The objectives of this lab activity are to:

  - Implement a synchronous counter circuit into physical circuit using Breadboard, Flip-Flops, Basic Gates and Switches.

  - Completing the next-state table of the counter circuit.

  - Sketch the state diagram of the counter circuit.

  - Identify the properties of the counter.

# Materials And Equipment

Materials and equipment required for this lab are as follows:

| Item Name | Number of Item |
| --- | --- |
| 1. Breadboard | 1 |
| 2. 7408 Quad 2-Input AND | 1 |
| 3. 7404 Hex Inverter | 1 |
| 4. 7432 Quad 2-input OR | 1 |
| 5. 7476 Dual J-K Flip Flop | 1 |
| 6. ETS-5000 Digital Kit | 1 |

# Preliminary Works

  - Determine the logic level for each input combinations in Table 1 so that the desired result can be realized.

Table 1

| Desired Result | ̅𝑃̅̅𝑅̅̅𝐸̅ | ̅𝐶̅̅𝐿̅̅𝑅̅ | J | K | CLK | Q |
| --- | --- | --- | --- | --- | --- | --- |
| Set initial value Q = 1 |  |  | X | X | -- |  |
| Output Q stays the same | 1 | 1 | 0 | 0 | ⇓ | 1 |
| Output Q become 0, no change in asynchronous input | 1 | 1 | 0 | 1 | ⇓ | 0 |
| Output Q is not the previous Q | 1 | 1 | 1 | 1 | ⇓ | 1 |
| RESET Q | 1 | 1 | 0 | 1 | ⇓ | 0 |
| SET Q | 1 | 1 | 1 | 0 | ⇓ | 1 |

  - Answer all questions.

- Which state that JK flip-flop has, but not on SR flip-flop.

TOGGLE (if both the value of J and K is 1)

- Identify whether the JK flip flop in 7476, is a positive-edge triggered or negative- edge triggered flip flop.

# It is Negative-Edge triggered flip flop, because has a bubble on its Clock input.

Count Down

Lab Activities

- You are given a counter circuit as shown in Figure 4.

Figure 4: A Synchronous Counter Circuit

- By using all materials and equipment’s listed in section C, construct the physical circuit of Figure 4. (Make sure all ICs are connected to Vcc and GND).

- Investigate the behaviour of the counter by observing the next state of the counter for all combination of Present State and X values. Complete the NextState table of the counter in Table 2. Ensure the Switch 0 is in HIGH state.

(0=LOW, 1=HIGH)

Count

Up

Table 2

| Switch 7 | Present State |  | Next State |  |
| --- | --- | --- | --- | --- |
| X | Q1 LED 1 | Q0 LED 0 | Q1 LED 1 | Q0 LED 0 |
| 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 |

- By referring to the Next-State in Table 2, sketch the state diagram of the counter.

- By referring to the Next-State in Table 2 and the state diagram in (4), answer all questions.

  - What is the main indicator to decide that the counter is a synchronous counter?

# Both Switch 0 (CLR’) and Switch 1 (PRE’) are high.

  - How many states are available for the counter and what are they?

# There are four (2^2), 00, 01, 10, and 11.

  - What is the function of Switch 7 (X) in the circuit?

# To determine whether its Count Up or Count Down

  - What is the function of Switch 0 and Switch 1 in the circuit?

# To set CLR’ (Switch 0) and PRE’ (Switch 1) whether its synchronous or asynchronous mode.

  - Is the counter a saturated counter or recycle counter?

# Saturated Counter, because it maintain the highest and the lowest value.

- Referring to state diagram in 4, draw and built a synchronous counter using D flip-flop.

  - Built the next state and transition table using the header in Table 3

Count

Up

Table 3

| Input X | Present State |  | Next State |  | D FF Transition |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Q1 | Q0 | Q1+ | Q0+ | D1 | D0 |
| 0 | 0 | 0 | 0 | 1 | 0 | 1 |
| 0 | 0 | 1 | 1 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 | 1 | 1 | 1 |
| 0 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 | 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 | 1 | 0 |

Count Down

Get the optimized Boolean expression.

| D1 | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 1 | 0 |

D1 = (X’.Q0)+(X’.Q1)+(Q1.Q0)

| D0 | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 1 | 1 |
| 1 | 0 | 0 | 0 | 1 |

D0 = (X’.Q1)+(Q1.Q0’)+(X’.Q0’)

C) Draw the complete final circuit in Deeds.

- Repeat steps in Q(6) using T flip-flop.

  - Built the next state and transition table using the header in Table 3

Count

Up

Table 3

| Input X | Present State |  | Next State |  | T FF Transition |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Q1 | Q0 | Q1+ | Q0+ | T1 | T0 |
| 0 | 0 | 0 | 0 | 1 | 0 | 1 |
| 0 | 0 | 1 | 1 | 0 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 | 1 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 | 1 | 1 |
| 1 | 1 | 1 | 1 | 0 | 0 | 1 |

Count Down

Get the optimized Boolean expression.

| T1 | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 |

T1 = (X’.Q1’.Q0)+(X.Q1.Q0’)

| T0 | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 1 | 1 | 1 |

T0 = (X’.Q1’)+(X.Q0)+(Q1.Q0’)

C) Draw the complete final circuit design in Deeds.